# PPE-Page 5                                  FIREFIGHTER I

- FIELD REDUCTION OF CONTAMINANTS

- DONNING

# ROPES-Page 5

- HOISTING TOOLS

- KNOTS

- MAINTENANCE

# RADIO AND PHONES-Page 6

- FIRE OR EMERGENCY CALL

- BUSINESS OR PERSONAL CALL

- OPERATING

# SCBA-Page 6-9

- IDENTIFY COMPONENTS

- DONNING

- EMERGENCY PROCEDURES

  - SKIP BREATHING

  - FACE PIECE FAILURE

INTERRRUPTION/RESTRICTED AIR FLOW

FIREFIGHTER I

  - EVENT OF FREE FLOW AIR

  - EXHALATION VALVE SEARED SHUT

- DOFF AND CHECK SCBA

  - WITH CYLINDER EXCHANGE

- MAINTENANCE

# EMERGENCY RESPONSE – APPARATUS-Page 10

- APPARATUS SAFETY

# FORCIBLE ENTRY-Page 10-12

- MAINTANENCE

- DOORS

  - SWINGING

  - SLIDING

  - OVERHEAD

- WINDOWS

  - SLIDING

  - DOUBLE HUNG

FORCIBLE ENTRY CONTINUED-Page 10-12

- WALLS

  - MASONRY

  - METAL

  - WOOD

# SAFETY-Page 12-13

- EXIT THROUGH A RESTRICTED PASSAGE

- Without removing SCBA

- Removing SCBA

- TRAPPED IN A FIRE OR HOSTILE SITUATION

  - USING HOSE LINE

  - USING A WALL

# LADDERS-Page 14-15

- 1 MAN SINGLE/ROOF LADDER

- EXTENSION LADDERS

- WORKING WITH A TOOL

- BRINGING A VICTIM DOWN

- ROOF LADDER ON A PITCHED ROOF

- MAINTENANCE

# HOSE, NOZZLES, APPLIANCES-Page 15-18

- MASTER STREAM

- EXTENDING A CHARGED HOSELINE

- REPLACING A BURST SECTION

- COUPLING AND UNCOUPLING

  - FOOT-TILT METHOD

  - TWO FIRE FIGHTER METHOD

- ADVANCE A CHARGED/UNCHARGED ATTACK LINE

  - FIRE ATTACK GROUND LEVEL

  - GROUND LADDER TO SECOND FLOOR WINDOW OR ROOF

  - OUSIDE STAIRWAY TO UPPER FLOOR

- STANDPIPE CONNECTION AND ADVANCE

- HOSE CARRIES

  - SINGLE SECTION DRAIN AND CARRY

  - ACCORDION SHOULDER CARRY

- HOSE LOADS

  - FLAT/ACCORDION LOAD SUPPLY LINE

  - FLAT/TRIPLE FOLD/MINUTEMAN ATTACK LINE

- HOSE ROLLS

  - STRAIGHT ROLL

  - DONUT ROLL

# HOSE, NOZZLES, APPLIANCES CONTINUED

- MAINTAINENCE

  - FIRE HOSE AND COUPLINGS

  - NOZZLES

# SEARCH AND RESCUE-Page 19

- VICTIM SEARCH

- RESCUE PROCEDURES

  - EXTREMITIES CARRY

  - COAT DRAG

VENTILATION-Page 20

FIREFIGHTER I

- BREAKING WINDOW OR DOOR GLASS

- VERTICAL VENTILATION

- ROOF

- FLOOR

- HIGH VOLUME BLOWER FAN

# OVERHAUL OPERATIONS-Page 21

- HIDDEN FIRES

# SALVAGE OPERATIONS-Page 21-23

- FOLDING SALVAGE COVER

- DEPLOYMENT OF SALVAGE COVER

  - ONE FIREFIGHTER

  - TWO FIREFIGHTER

- WATER CHUTE

- CATCHALL

- COVERING BUILDING OPENINGS

- SEPARATE CHARRED FROM UN-CHARRED MATERIAL

- STOPPING A SPRINKLER HEAD

- OPERATE A MAIN CONTROL VALVE

- MAINTENANCE OF SALVAGE COVERS

# WATER SUPPLY-Page 23

- HYDRANT TO PUMPER HOSE CONNECTION

  - FORWARD HOSE LAY

  - REVERSE HOSE LAY

WATER SUPPLY CONTINUED Page-23-24

- HAND LAY 300 FT OF SUPPLY LINE

- DRAFTING (STATIC WATER)

- DRAFTING (PORTABLE TANK)

# PORTABLE FIRE EXTINGUISHERS-Page 24

- USE FOR (CLASS A, B, C) FIRES

# FIRE GROUND SUPPORT ACTIVITIES-Page 25

- LIGHTING EQUIPMENT

- UTILITY SERVICES

  - PROPANE TANK OR NATURAL GAS VALVE

MAIN POWER BREAKER

FIREFIGHTER I LIVE EVOLUTIONS-Page 26

- VEHICLE FIRE

- CLASS A FIRE EXTERIOR

- OFFENSIVE INTERIOR FIRE ATTACK

- GROUND COVER FIRE

- VERTICLE VENTILATION

EXAMINATION EVOLUTIONS-Page30-33

- RESCUE EVOLUTION

- SALVAGE & OVERHAUL

- DEFENSIVE OPERATIONS

- LADDER EVOLUTION

FIREFIGHTER II SKILLS –Page 34

HAZARDOUS MATERIAL-Page 41

FIREFIGHTER I

PPE

1-PPE FIELD DECONAMINATION

- Doff SCBA

- Verbalize the use of hoseline to rinse off all PPE and equipment

- Doff PPE

- Verbalize the need to bag contaminated PPE

- Verbalize the need for formal cleaning - Inspect all equipment for damage

5 minutes

2- Donning PPE

- Don hood (hood may be rolled down around collar of coat)

- Don and fasten all closures on pants/boots and coat

- Collar turned up on coat/hood rolled on collar

- Don helmet and tighten chin strap (helmet ear flaps down)

- Don gloves

- Doff protective clothing and place back in-service

1 minute (No time limit for doffing)

ROPES

3- Hoisting tools: Fire axe, pike pole, roof ladder, hose

- Select tool to be hoisted

- Tie with an approved knot for hoisting

- Use ‘tag’ line when directed

- Hoist tool or equipment

1 Minute (Hoisting time not included in the 1 minute)

4-Tie all knots per knot sheet

5- Inspecting, cleaning, and storing rope after use:

- Identify type of rope, life safety or utility

- Inspect rope for damage or wear

- Wash rope using approved method

- Dry rope using the approved method

- Bag or coil rope

- Document the use of the rope and inspection on rope log

3 Minutes per 50 feet( Coiling or bagging rope)

RADIO AND PHONES

6- Report of a fire or emergency:

- Identify self

- Record time of call

- Record type and address of incident

- Record callers name and call back number

- Ask questions for additional information - Advise caller to call 911

- Hang up after caller - Relay message to dispatch

1 Minute

7- Receiving a business or personal call from public:

- Identify self

- Record time of call

- Record message

- Record callers name and call back number

- Hang up after caller - Relay caller’s information

1 Minute

8- Use of mobile and portable radio equipment:

- Turn on power

- Set to correct channel

- Adjust volume

- Adjust squelch (If radio is equipped)

- Perform radio check (Simulate)

1 Minute

SCBA

9- Identify and describe each SCBA component:

- Backpack/harness

- Air cylinder- cylinder, valve, and pressure gauge

- Regulator assembly- high pressure hose, low pressure alarm, main line valve, emergency by-pass, secondary gauges

- Face piece assembly- low pressure hose, exhalation valve and head harness

- PASS device

5 Minutes

10- Don SCBA:

- Crouch or kneel at SCBA (if donned from ground).

- Check SCBA cylinder pressure gauge (Must verbalize pressure)

- Open cylinder valve fully

- Check regulator and cylinder gauge to match (Within 100psi)

- Don backpack and fasten/tighten all straps (Over-the-head or coat method)

- Don face-piece adjust head harness, check seal and exhalation valve

- Don protective hood (No hair or skin showing) - Don Helmet with chin strap secured

- Attach low pressure hose to regulator or face-piece

- Ensure PASS is activate - Don gloves (No skin showing)

1 Minute

11 Demonstrate emergency procedures in the event of SCBA failure:

A- Skip breathing

- Inhale normally (As during regular breathing)

- Hold breath (As long as it would take to exhale)

- Inhale again

- Exhale slowly

1 Minute

B- Emergency procedures for face piece failure:

- Notify partner of problem

- Activate emergency communications (Mayday)

- Activate PASS - Exit structure with partner

1 Minute

C- Emergency procedures for interruption/restricted air flow:

- Notify partner of problem

- Activate emergency communications (Mayday)

- Activate PASS

- Operate by-pass: 1. Open By-pass valve, inhale 2. Close By-pass valve, 3. Repeat as needed.

- Begin to exit structure with partner

1 Minute

D- Emergency procedure in the event of Free Flow air

- Notify partner of problem

- Activate emergency communications (Mayday)

- Activate PASS

- Exhale forcefully to reset valve or regulator

- If exhaling does not resolve problem, open and close By-pass valve once

- If By-pass valve doesn’t resolve problem, restrict flow of air out of air cylinder by closing bottle valve, and opening as needed

- Crack bottle open, take breath, close bottle, repeat as needed

- Begin to exit structure with partner

1 Minute

E- Emergency procedure in the event of Exhalation valve being ‘seared’ shut

- Notify partner of problem

- Activate emergency communications (Mayday)

- Activate PASS

- Hold face-piece in place and exhale forcefully

- If exhaling does not resolve problem - Exit the structure with partner

1 Minute

12- Doff and perform a check of SCBA after use with a cylinder change.

- Place cylinder on ground, close cylinder valve

- Obtain full cylinder

- Relieve excess pressure from regulator

- Disconnect high pressure hose

- Release clamp on bottle and remove cylinder from frame

- Check pressure on new bottle, verbalize pressure reading

- Place new cylinder in frame and secure clamp or closure

- Check high pressure O-ring

- Connect high pressure hose to cylinder

- Open cylinder valve and check for leaks

- Check cylinder and secondary gauges to be within 100psi

- Close cylinder valve and relieve excess pressure from regulator, listen for low-pressure alarm

- Fully extend all straps on SCBA backpack

- Fully extend all straps on face-piece

2 Minutes

13- Maintenance procedures for an in-service SCBA

- Inspect SCBA and face piece for flaws

- Wash face piece and sanitize (Verbalize sanitation procedure)

- Rinse in fresh water, hang to dry

- Wash/rinse backpack (Keep water from entering regulator)

5 Minutes

EMERGENCY RESPONSE – APPARATUS

14- Demonstrate safety procedure for mounting and dismounting fire apparatus, use of seat belts, hearing protection, and other safety equipment on apparatus.

- Use handrails, steps to mount apparatus

- Properly fasten seat belts

- Don hearing protection if needed

- Uses handrails or steps to dismount apparatus

2 Minutes

FORCIBLE ENTRY

15- Demonstrate proper care and maintenance of forcible entry tools and equipment (All cutting, prying, pulling, or striking tools)

- Carry tool in safe manner

- Protect self and others from sharp or pointed edges during maintenance

- Remove excess dirt with brush or hose

- Wash tool with soap and water

- Inspect tool head and handle for damage

- Sharpen tool heads (Per manufacturer recommendation or department SOP)

- Dry tool and protect surface as per manufacturer recommendations.

5 Minutes

16- Demonstrate forcible entry through-the-lock entry techniques for the following doors:

A- Swinging doors (stopped or rabbeted jamb)

- Select proper forcible entry tool

- Carry tool in a safe manner

- Try before you pry

- Correctly identify door type (Stopped or Rabbeted)

- Pry or force entry at or near lock

- Force door in the correct direction - Control door and access

1 Minute

B- Sliding Doors

- Select proper forcible entry tools

- Carry tool in a safe manner

- Try before you pry - Correctly identify door type (stopped or rabbeted).

- Pry or force entry at lock or near bottom of door - Force door away from frame - Control door and access

1 Minute

C- Overhead doors

- Select proper forcible entry tools

- Carry tool in safe manner

- Try before you pry

- Force entry with appropriate cut - Control the door and access

2 Minute

17- Demonstrate forcible entry and through-the-lock entry techniques for the following windows:

A- Sliding windows

- Select proper tools

- Carry tool in safe manner

- Try before you pry

- Locate lock

- Pry or force entry at proper location on window - Clear away window coverings and obstructions

1 Minute

B- Double hung windows

- Select proper tools

- Carry tool in safe manner

- Try before you pry

- Locate lock

- Pry or force entry at center of lower sash - Clear away window coverings and obstructions

1 Minute

18- Demonstrate forcible entry for the following exterior walls

A- Masonry

- Select proper tools

- Carry tool in a safe manner - Confirm utilities are off

- Correctly use tools to force entry

- Identify possible hazards associated with breaching wall

5 Minutes

B- Metal

- Select proper saw or tools

- Carry tool in a safe manner

- Confirm utilities are off

- Find and mark studs or supports (if applicable)

- Make appropriate cut

- Identify possible hazards associated with breaching wall

5 Minutes

C- Wood

- Select proper tools

- Carry tool in a safe manner

- Confirm utilities are turned off

- Remove exterior covering (if needed)

- Sound for studs or supports

- Cut along studs

- Identify possible hazards associated with breaching wall

5 Minutes

SAFETY

19- Demonstrate the use of SCBA to exit through:

A- Restricted passage without removing SCBA

- Notify Command of situation

- Loosen SCBA backpack (Leave face-piece on) - Maintain control of SCBA at all times

- Move SCBA backpack to side of firefighter

- Exit restricted passage

- Reposition and tighten SCBA backpack

3 Minutes

B- Restricted passage removing SCBA

- Notify Command of situation

- Move SCBA backpack completely

- Maintain control of SCBA at all times - Exit through restricted passage - Re-don backpack assembly

- Tighten SCBA backpack

3 Minutes

20- Demonstrate the techniques for action when trapped or disoriented in a fire or hostile situation:

A- Search for an exit using a hose line (2-firefighter team)

- Notify command of situation (Mayday)

- Communicate to team members actions, problems or needs for assistance

- Activate PASS device

- Attempt to retrace steps

- While searching for an exit, locate hose, find coupling

- Use hose and follow to safety using coupling as a guide

- Maintain contact with team members verbally, physically or sight

- Exit hazardous area before exhausting air supply as a team

3 Minutes

B- Search for an exit using a wall (2-firefighter team)

- Notify command of situation (Mayday)

- Communicate to team members actions, problems or needs for assistance

- Activate Pass device

- Attempt to retrace steps

- Search for wall

- Find wall and follow to safety using right or left-handed technique

- Maintain contact with team members verbally, physically or sight

- Exit hazardous area before exhausting air supply as a team

2 Minutes

LADDERS

21- Demonstrate the proper carry, position, raise, and lower of a roof ladder

- Start 25 feet from a structure

- Lift ladder with legs (Not back) to carry position

- Carry ladder correctly (Tip or heal forward according to ladder type)

- Check for overhead obstructions (Must verbalize)

- Determine structure stability (Must verbalize)

- Butt and raise ladder to proper position

- Check climbing angle

- Lower ladder to ground

1 Minute (Lowering ladder not included in time)

22- Demonstrate the proper carry for a 24' (2-firefighter team) or 35' (4 firefighter team) extension ladder

- Start 50' from structure

- Leader gives predatory commands

- Team lifts with legs

- Proper carry to structure

- Check for overhead obstructions (Must verbalize)

- Determine structure stability (Must verbalize)

- Raise fly section to proper height

- Determine locks are engaged (Must verbalize)

- Tie off halyard

- Check climbing angle

- Lower ladder to ground

3 Minutes (Lowering ladder not included in time)

23- Demonstrate procedure for climbing, working, and descending from extension or aerial ladder with a tool (with safety harness for aerial ladder)

- Check climbing angle of ladder

- Climb with arms straight, body perpendicular to the ground

- Climb smoothly and rhythmically

- Maintain three points of contact with ladder while climbing

- Carry tool properly

- Lock in around rung or use safety harness

- Lock in on opposite side from the working side

2 Minutes

24- Demonstrate bringing a victim down a ladder from a 2nd story window or roof

- Ensure tip of ladder placed at the sill of the rescue window (Must verbalize)

- Proper carry of victim (Conscious and unconscious) - Descend ladder one rung at a time reassuring the victim

5 Minutes

25- Demonstrate deployment of a roof ladder on a pitched roof (2-firefighter team)

- Raise roof ladder and place against extension ladder or roof

- Properly climb extension ladder

- Carry roof ladder to the roof line and lock-in

- Slide roof ladder up roof

- Ensure hooks are securely grabbing peak of roof

2 Minutes

26- Demonstrate inspection, maintenance, and cleaning procedures for ground and/or aerial ladders (2-firefighter team)

- Wash ladder with water, soap, brush, and then dry ladder

- Check rungs for looseness, cracks, dents, or unusual wear

- Check all bolts, rivets, and welds for looseness

- Check beams, trusses, and truss blocks for damage

- Check all braces, slides, stops, locks

- Check pulleys and halyards

- Check heat sensor labels for color change

- Mark any defects found and tag out of service

- Check for smooth operation by raising and lowering ladder - Record cleaning, inspections and maintenance

10 Minutes

HOSE, NOZZLE, APPLIANCE

27- Demonstrate proper set up of a master stream (100ft, 2½” hose or larger) (300 gpm or higher) (2-firefighter team)

- Set up appliance

- Connect hose line from apparatus to appliance

- Secure monitor as per manufacturers guidelines

- Signal for water

- Exercise safety as hose line charges

- Adjust stream of appliance once water is at nozzle

- Flow water

5 Minutes

28- Demonstrate the proper method for extending a charged hose line (2-firefighter team)

- Clamp hose approximately 3-5 feet behind nozzle or communicate to shut down of hoseline, remove nozzle

- Add 50’ section of hose

- Replace nozzle and charge hoseline slowly

1:30 Minute

29- Demonstrate the proper method for replacing a burst section of hose (2-firefighter team)

- Clamp hose approximately 3-5 feet behind coupling or communicate to shut down of hoseline

- Drain hose line at nozzle

- Replace 2 sections of hose for every one burst section

- Remove hose clamp and charge hoseline slowly

2 Minutes

30. Demonstrate coupling and uncoupling hose using the following

A. Foot-tilt method

- Place foot behind male end of hose and apply pressure to tilt male coupling - Check the gasket

- Connect female end to male end

20 Seconds

B. Two fire fighter method (2-firefighter team)

- One fire fighter holds male coupling and one fire fighter holds female coupling - Check the gasket

- Fire fighter holding female coupling couples’ hose to male coupling using the Higbee indicator as a guide

20 Seconds

31. Advance a charged and uncharged attack line from a pumper, for the following using 150’ of 1 ½ minimum

A. For ground level fire attack (structural, ground cover, vehicle) (2-firefighter team)

- Correctly unload hose from apparatus

- Advance hose to attack position, extend hose line to avoid kinks

- Signal for water

- Open nozzle to bleed air from attack line and check stream pattern

2 Minutes

B. Up a ladder to second floor window or roof (2-firefighter team)

- Correctly unload hose from apparatus

- Advance uncharged hose line up the ladder

- Secure hose to the ladder with hose straps or webbing - If working from ladder, ensure proper leg lock or safety harness

- Signal for water

- Open nozzle to bleed air from attack line and check stream pattern

3 Minutes

C. Up/down an inside/outside stairway to upper/lower floors (150ft of 1½” hose or larger) ( 2-firefighter team)

- Correctly unload from apparatus

- Advance to door

- Signal for water

- Open nozzle to bleed air from attack line and check stream pattern

- Advance up or down 2 flights of stairs

- Lay hose against outside wall

3 Minutes

32. Demonstrate how to connect to a standpipe and advance a hoseline from the standpipe (100ft of 1½” or larger hose) (2-firefighter team)

- Advance to standpipe with needed equipment

- Connect to standpipe at connection below fire floor

- Properly position hoseline in stairwell

- Advance the entire hose length, avoiding kinks

- Signal for water

- Open nozzle to bleed air from attack line and check stream pattern

2 Minutes

33. Demonstrate the following hose carries:

A. Single section drain and carry

- Pick-up coupling allowing water to drain

- Place coupling in front of body with hose loop over shoulder

- Layer hose on shoulder

- Hose controlled and secured

1 Minute

B. Accordion shoulder carry

- Fold hose on ground

- Line up folds and couplings

- Lift hose to shoulder using legs

- Hose controlled and secured

1 Minute

34- Demonstrate the following hose loads:

A. Load supply hose lines on fire apparatus using the flat or accordion hose load (150ft of 2½” or larger hose)(3-firefighter team)

- Layer hose in hose bed correctly

- Use Dutchman where appropriate

3 Minutes

B- Load attack hose lines on fire apparatus using the Flat, Triple fold, or Minuteman hose load (150ft of 1½” or 1¾” hose)(3-firefighter team)

- Attach female coupling to discharge

- Layer hose in hose bed correctly

- Use Dutchman where appropriate

- Finish load with nozzle in correct location and secured

3 Minutes

35- Demonstrate the following hose rolls

A- Straight roll

- Roll hose with male coupling in the center

- Lay completed roll on ground, tamp protruding coils 1 Minute

B- Donut Roll

- Roll hose with male coupling inside roll and female coupling approximately 3' ahead of male coupling

- Lay completed roll on ground, tamp protruding coils

1 Minute

36- Demonstrate the procedures for cleaning, inspecting and maintaining:

A- Fire hose and couplings

- Wash hose using clear water and a brush, if required use mild soap

- Rinse with clear water

- Wash couplings with clear water and brush

- Inspect hose for mechanical, thermal, or chemical damage

- Inspect couplings for damage

- Remove gasket from female coupling and pinch together, check for cracks

- Reinstall gasket in female coupling

- Check male threads for damage by threading into female coupling

- Mark and record any damage found, if necessary, remove from service

5 Minutes

B- Nozzles combination, fog, and smooth bore

- Wash nozzle using mild soap and water

- Rinse with clear water

- Inspect nozzle for external damage or debris

- Inspect nozzle for internal damage or debris

- Remove gasket from nozzle and check for cracks

- Reinstall gasket

- Check female swivel and place nozzle on hose

- Demonstrate proper operation of nozzle

4 Minutes

SEARCH AND RESCUE

37- Demonstrate proper techniques while conducting a search for a victim in a structure and remove victim to a safe area (2-firefighter team)

- Stay low while searching

- Organized search pattern, maintaining orientation

- Call out for victim

- Pause occasionally to listen for sounds

- Locate victim and communicate it to command

- Remove victim to safe area using a appropriate drag or carry

- Team integrity is maintained during search

6 Minutes per 300 sq. ft. searched

38- Demonstrate proper rescue procedures for the following:

A- Move a victim 20 feet using the extremities carry (2-firefighter team)

- Lift with legs

- One firefighter behind the patient, reaching underarms, grabbing wrists

- Second firefighter facing the same direction slips hands under knees

- Move victim 20 feet upon command of firefighter at head

1 Minute

B- Move an unconscious fire fighter with or without functioning SCBA, 20 ft. using the coat drag (2-firefighter team)

- Place fire fighter face-up

- Grasp fire coat by the collar, SCBA strap, or integrated drag strap

- Work as a team to move the fire fighter to a safe area

- Move fire fighter 20 ft.

1 Minute

VENTILATION

39- Demonstrate procedures for breaking window or door glass and removing obstructions, from ground level and while working from a ladder:

- Stand or ladder to the windward side of the glass pane, leg lock on ladder

- Strike with tool at the top of the pane

- Keep hands above or aside point of impact

- Stand at 45-degree angle to window/door if using pike pole

- Clear all glass from frame or track

- Remove screens and window coverings

1 Minute

40 Demonstrate opening a 4' x 4' ventilation hole for all of the following for vertical ventilation:

A-Pitched or Flat roof (3-firefighter team)

- Proper use of roof ladder (Wind is at firefighters back or side)

- Use spotter for safety

- Sound roof for integrity - Correctly open ventilation hole

- Use appropriate tool to push down ceiling and remove any barriers

- Exit roof area once ventilation hole is completed

4 Minutes with power tools (Time starts when cut begins)

6 Minutes with hand tools (Time starts when cut begins)

B- Floor (2-firefighter team)

- Sound floor for structural supports

- Correctly open ventilation hole

- Use appropriate tool to remove any barriers

- Exit once ventilation hole is completed

4 Minutes with power tools (Time starts when cut begins)

6 Minutes with hand tools (Time starts when cut begins)

41- Demonstrate the setup of a high-volume blower fan in a doorway for positive pressure ventilation of a structure (2-firefighter team)

- Determine access point and fire location - Create appropriate exhaust opening a close to the seat of the fire as possible - Start positive pressure blower

- Set blower so that the cone of the air from the blower completely cover the door opening - Open door to structure and pressurize the building

- Set blower so placement does not interfere with entry & exit of firefighters from the building

- Exhaust opening should be 75 to 150 percent as large as the entry opening

- Open interior doors systematically to maintain positive pressure in the structure - Maintain team integrity during ventilation work

3 Minutes

OVERHAUL OPERATIONS

42- Demonstrate search and exposure of hidden fires in: Walls, ceilings, and floors

- Search for hidden fires using sight, touch, sound, and thermal sensors

- With appropriate tool open walls, ceilings and floors to expose hidden fire

- Expose void spaces without compromising structural integrity

- Use water as needed to extinguish hidden fires as found (Protect from water damage)

- Fire cause evidence is preserved

5 Minutes

SALVAGE OPERATIONS

43- Demonstrate the proper folds and rolls for salvage covers

Fold or roll a salvage cover for a one or two fire fighter spread

- Fold cover to reduce size - Fold with finished side out - Finish cover by rolling or folding (Two firefighter team – 1 Minute) 44-Demonstrate 0ne-firefighter and two-firefighter deployment of a salvage cover A.) One fire fighter deployment. - Stack and gather objects to be covered - Unfold or unroll cover over all objects - Tuck covers in at the bottom. (One firefighter team – 1 Minute). (Two firefighter team – 1 Minute) B.) Two firefighter deployment - Stack objects to be covered - Stretch cover alongside of the object to be covered - Make several accordion folds with inside hand - Pull the cover tight and throw, pocketing as much air as possible - As team float the cover over the object to be covered - Guide into position and tuck in at the bottom 1 minute..

45- Demonstrate the proper construction of a water chute (2-firefighter team)

- Open salvage cover

- Roll salvage cover edges over pike poles and turn over to create a water chute

- Adjust water chute on ladder for water removal out a window

- Remove water from structure

2 Minutes

46- Demonstrate the proper construction of a catchall (2-firefighter team)

- Open salvage cover

- Roll side inward approximately 3'

- Fold ends of the side rolls at a 90-degree angle

- Tuck the end roll under the side roll to lock

2 Minutes

47- Demonstrate the covering or closing of a building opening including: doors, windows, floors, and roofs (2-firefighter team)

- Select correct materials to cover openings

- Use sufficient amount of material to cover the opening completely

- Secure edges of material down completely to keep weather out

5 Minutes

48- Separate, remove, and relocate charred material from un-charred material while protecting the area of fire origin for cause determination (2-firefighter team)

- Separate charred from un-charred material in cooperation with a fire investigator

- Extinguish any smoldering material

- Remove charred material to a safe area

- Preserve any evidence by leaving where found - Advise fire investigator where you found the evidence - Verify atmosphere is safe by operating air monitoring equipment - Verbalize the need for mitigation of hazard

5 Minutes

49- Demonstrate stopping the flow of water from a discharging sprinkler head, using a wedge or stopper

- Select the appropriate tool

- Stop sprinkler flow

1 Minute

50- Operate a main control valve on an automatic sprinkler system from ‘open’ to ‘close’ and then back to ‘open’

- Identify main control valve

- Operate the valve

1 Minute

51- Demonstrate the procedure of inspection, cleaning, and maintaining salvage covers

- Demonstrate washing and drying salvage cover

- Inspect cover for tears or holes

- Mark holes if found and patch according to manufacturer or departmental guidelines

2 Minutes

WATER SUPPLY

52- Demonstrate hydrant-to-pumper hose connection for a:

A- Forward hose lay

- Select correct equipment to connect to hydrant

- Loop hose around hydrant, secure hose (Stand on correct side of hydrant)

- Signal apparatus to proceed

- Connect hydrant gate valves

- Open hydrant completely and flush (open gradually to prevent water hammer)

- Charge hoseline (slowly) when signal is given

- Shut hydrant down slowly to prevent water hammer and check drain

3 Minutes (Time does not include shutting down)

B- Reverse hose Lay (2-firefighter team)

- Select equipment needed to place one attack hose in service and drop at scene (150')

- Remove and ground 50' of supply hose from apparatus

- Kneel on hose and signal apparatus to proceed

- Connect attack hose to supply hose, and signal for water when ready

- Charge hose line and open nozzle

3 Minute

53- Demonstrate a hand lay of 300 feet of supply line, 2 ½” or larger from a pumper to a water source (4-firefighter team)

- Shoulder load 50' of hose from hose bed, approximately 50’ between firefighters

- Lay entire length with no kinks or tangles

- Connect to hydrant

5 Minutes

54- Demonstrate the assembly and connecting of the equipment necessary for drafting for static water supply source (2-firefighter team)

- Fasten rope to strainer to aid in handling hard suction hose - Check for the gaskets

- Couple strainer to hard suction hose

- Couple the two section of hard suction hose together, taking care to not get dirt in coupling

- Connect completed hose to apparatus

- Keep a minimum of 24” of water surrounding the strainer

- Use a rubber mallet to ensure an air-tight connection

5 Minutes

55- Demonstrate the assembly and connecting of the equipment necessary for drafting from a portable water tank

- Set up portable water tank

- Fasten rope to strainer to aid in handling hard suction hose - Check the gaskets

- Couple strainer to hard suction hose

- Couple the two sections of hard suction hose together, taking care to not get dirt in coupling

- Connect completed hose to apparatus

- Use rubber mallet to ensure an air-tight connection

12 Minutes

PORTABLE FIRE EXTINGUISHER

56- Demonstrate the use of portable fire extinguisher on a Class A, Class B, and Class C fires

- Select correct extinguisher for type of fire

- Check pressure gauge

- Approach fire from upwind/upgrade - Use the PASS technique -Pull pin and squeeze the handle to test the extinguisher -Aim at the base of the fire -Squeeze the handle -Sweep side to side

1 Minutes

FIRE GROUND SUPPORT ACTIVITIES

57- Demonstrate safety procedures when using fire service lighting equipment

- Start power supply

- Reset ground-fault interrupter (GFI) if necessary

- Extend cord and light

- Illuminate light

2 Minutes

58- Demonstrate shutting of utility services to a building

A- Shut off propane tank or natural gas valve

- Select correct tool - Assess the scene for hazards

- Turn valve(s) to the off position

30 Seconds

B- Secure power to structure by shutting off main power breaker

- Select correct breaker - Assess the scene for hazards

- Switch breaker to the off position

30 Seconds

FIREGROUND EVOLUTIONS

FIREFIGHTER I

# LIVE FIRE EVOLUTIONS

- OFFENSIVE ATTACK ON A PASSENGER VEHICLE FIRE-Page 27

- OFFENSIVE ATTACK ON A DUMPSTER FIRE-Page 27

- OFFENSIVE ATTACK ON A INTERIOR FIRE-Page 28

- OFFENSIVE ATTACK ON A GROUND FIRE-Page 28

- VENTALATION SUPPORT ACTIVITIES FOR A INTERIOR ATTACK-Page 29

  - VERTICAL VENTILATION

FIREGROUND EVOLUTIONS

1- Demonstrate an Offensive attack on a passenger vehicle fire (2-firefighter team)

- Deploy traffic control device - Identify automobile fuel type - Assess for fuel leaks - Completely unload attack line from apparatus - Open attack line and flush line - Set nozzle pattern between straight stream to narrow fog 30 to 60 degrees - Attack fire from uphill and upwind if possible - Approach vehicle from a 45-degree angle - Immobilize vehicle - Attack fire, staying away from vehicle tires and bumpers - Extinguish any ground fires - Move to fire inside of vehicle - Gain entry into compartment on fire - Overhaul all automobile compartments to find any hidden fires 4 Minutes (Time begins when crew dismounts apparatus)

2- Demonstrate an Offensive attack on a class A fire, Exterior fire, dumpster or pile of class A material (2-firefighter team)

- Completely unload attack line from apparatus - Open attack line and flush line - Set nozzle pattern between straight stream to narrow fog - Attack fire from uphill upwind if possible - Extinguish any ground fire, then move to fire inside dumpster or pile of class A material - Cool the approach side of the dumpster, if applicable - Overhaul to find any hidden fires using hand tools 4 Minutes (Time begins when crew dismounts apparatus)

3- Demonstrate an Offensive attack on an Interior Class A fire, tank water as water source (2-firefighter team)

- Completely unload attack line from apparatus - Select forcible entry tool and carry to door - Secure hose - Signal for water - Open attack line and flush line - Set nozzle to appropriate pattern - Don face piece, go on air, check partner’s PPE (no skin showing, pass device activated) - Force entry on door (check door for heat, try before you pry) - Control the door and access - Check overhead in room, sound floor for integrity - Advance into structure searching for fire and possible victims (back up firefighter carries Forcible entry tool) - Find seat of fire and extinguish using correct nozzle pattern. - Maintain team integrity during attack (sight, voice, or physical contact) - Overhaul to find any hidden fires - Contact command; verbalize “fire under control” - Provide CAAN report. 5 Minutes (Time begins when crew dismounts apparatus)

4- Demonstrate an Offensive attack on a Class A fire, Ground cover fire (2-firefighter team)

- Completely unload the attack line from apparatus (if necessary) - Attack fire from the blackened side of fire - Attack fire from downhill and upwind if possible - Observe weather conditions for changes - Maintain team integrity during attack 5 Minutes (Time begins when crew dismounts apparatus)

5- Demonstrate vertical ventilation activities in support of fire attack operations on an interior structure fire:

- Determine location for ventilation hole - Select appropriate tools. - Start chain saw on ground (if used) - Remove extension ladder from apparatus and carry to access point - Properly position extension ladder and raise to roof - Properly climb ladder - Properly place roof ladder on roof - Determine roof integrity by sounding roof - Cut inspection hole or safety/Kerf cuts as neccesary - Hoist or carry tools and equipment to roof - Position second ladder as an emergency escape route, away from fire - Select correct location above fire to cut hole while working from roof ladder - Cut ventilation hole - Coordinate with command and interior crews to open hole - Maintain team integrity during ventilation work - Exit roof area once ventilation hole has been cut - Notify command ventilation is complete 8 Minutes (Time begins when crew dismounts apparatus)

EXAMINATION EVOLUTIONS

- RESCUE EVOLUTION 2-member team

- Use handrails and steps to dismount apparatus

- Correctly and completely unload hose from apparatus

- Select proper forcible entry tool(s)

- Carry tool in a safe manner

- Advance hose to attack position, extend hoselines to avoid kinks

- Secure hose

- Signal for water

- Open nozzle to bleed air from attack line and check stream pattern

- Don face piece, go on air, check partner’s PPE (no skin showing, pass device activated

- Correctly identify door type

- Try door before prying

- Pry or force entry near lock

- Force door correct direction

- Control the door and access

- Sound floor for integrity, check overhead in room

- Make entry searching for a victim with or without a hoseline

- Conduct a search

- Organized search pattern, maintaining orientation

- Call out for victim

- Pause occasionally to listen for sounds

- Locate victim and communicate it to command

- Move victim out of structure using appropriate techniques

- Maintain team integrity during evolution

6:00 minutes

2.) Salvage and Overhaul Evolution 2-member team

Don PPE

- Don hood (hood may be rolled down over collar of coat)

- Don and fasten all closures on pants/boots and coat

- Collar turned up on coat

- Check SCBA cylinder pressure gauge verbalize

- Open cylinder valve completely

- Check regulator and cylinder gauge to match within 100 psi

- Turn on or ensure PASS is active

- Don backpack and fasten/tighten all straps (over the head or coat method)

- Don face piece, adjust head harness, check seal and exhalation valve

- Protective hood pulled over head

- Attach hose to regulator or facepiece

- Don helmet with chin strap secured

- Don gloves

- All PPE donned appropriately with no skin or hair showing

Ventilate structure

- Start positive pressure fan

- Establish exhaust opening verbalize

- Set fan so that the cone of air completely covers the door opening

- Set fan so placement does not interfere with the entry or exit of firefighters

Remove water from structure by

- Build a catchall

- Open salvage cover

- Roll sides inward approximately 3’

- Fold ends of the side rolls at a 90-degree angle

- Tuck the end roll under the side roll to lock

- Build a water chute

- Open salvage cover

- Roll salvage cover edges over pike poles and turn over to create water chute

- Adjust water chute on ladder for water removal

- Communicate assignment complete to command

8:00 minutes

3.) Defensive Operations Evolution 2-member team

Secure Hydrant

- Use handrail and steps to dismount apparatus

- Correctly unload hose from apparatus

- Select correct equipment to connect to hydrant

- Connect hydrant gate valves

- Open hydrant completely, flush

- Charge supply hose slowly, when signal from engineer is given

Deploy a Master Stream 350 GPM or greater

- Set up appliance

- Connect 100 ft hoseline per inlet from apparatus to appliance

- Secure monitor as per manufactures guidelines AHJ

- Signal for water

- Exercise safely as hoselines charge

- Adjust stream of appliance once water is at nozzle

- Flow water to hit target

Extend a charged Hoseline with nozzle

- Apply hose clamp approximately 3-5 feet behind coupling, or communicate to shut down the hoseline

- Add 50’ section of hose

- Replace nozzle and charge hoseline slowly

# 4.) Ladder Evolution 2-member team

- Use handrails and steps to dismount apparatus

- Start 50’ from structure

- Leader gives preparatory commands

- Team lifts with legs

- Properly carry tools and ladder to structure

- Check for overhead obstructions

- Determine structure stability

- Raise fly section to proper height

- Determine locks are engaged must verbalize

- Tie off halyard

- Check climbing angle, four points of contact

- Ladder is secured prior to climbing

- Climb with arms straight, body perpendicular to the ground

- Climb smoothly and rhythmically

- Maintain three points of contact of ladder while climbing

- Carry tool properly

- Sound roof for integrity before stepping on

- Stay on roof supports must verbalize

- Hoist an axe or pike pole

- Tie with an approved knot for hoisting

- Use tag line when appropriate

- Lower ladder and tools safely to the ground (not included in time)

# 7:00 minutes

# FIREFIGHTER II

# GENERAL-Page 35

- ASSUMING AND TRANSFERRING COMMAND

# FIRE DEPARTMENT COMMUNICATIONS-Page 35

- FIRE REPORTS (UFIRS)

# FIREGROUND OPERATIONS-Page 35-37

- FOAM FIRE STREAM

- INTERIOR FIRE ATTACK

FIREFIGHTER II

GAS CYLINDER(LPG)

- PROTECT EVIDENCE AND FIRE CAUSE AND ORGIN

# RESCUE OPERATIONS-Page 37-38

- VEHICLE EXTRICATION

  - WINDSHEILD

  - ROOF

  - DOOR

  - DASHBOARD

# PREVENTION, PREPAREDNESS, & MAINTENANCE-Page 38-39

- PRIVATE DWELLING SURVEY

- STATION PRESNTATION

- PREINCIDENT SURVEY

- POWERPLANT AND LIGHTING EQUIPMENT MAINTENANCE

- FIRE HOSE AND SERVICE TEST

Examination Evolutions-Page 40

- Extrication Evolution

- Extinguishment of a Class B Fire

FIREFIGHTER II

GENERAL

1- Demonstrate procedures for assuming and transferring command at an emergency scene, utilizing an incident management system

- Give a brief size-up

- Unit on scene - Location of command

- Type of building and construction

- Current conditions (This is what I see)

- Declare a strategy

- Initial actions (What I need to do immediately) - Identify Side A

- Establish Incident Command & Accountability and announce location

- Name the incident

- Request additional resources if needed.

- Transfer of command (Utilizing department guidelines)

2 Minutes

FIRE DEPARTMENT COMMUNICATIONS

2- Complete a basic fire report by completing a Utah Fire Incident Report System (UFIRS)

- Complete all blanks/boxes for scenario

- Correct data and information

- Check spelling

- Legibility

- Other pertinent information is recorded in narrative

15 Minutes

FIREGROUND OPERATIONS

3- Demonstrate the correct procedures for extinguishment of an exterior combustible liquids fire with a foam fire stream (2-Firefighter team)

- Assemble foam application system, if applicable

- Select correct concentration for required application, if applicable

- Approach spill as part of coordinated team

- Demonstrate proper application techniques (Bank, Roll, or Rain)

- Retreat from spill maintaining team integrity

5 Minutes

4-Coordinate an interior attack line for team’s accomplishment of an assignment in a structure fire (2-firefighter team)

- Determine location of fire (Attic, grade level, upper level, or basement level)

- Receive 360 walk-around report

- Ensure overhead spaces are checked for fire

- Ensure the floor has been sounded

- CAAN (Conditions, Actions, Air, Needs) Report (Monitor smoke, heat, fire)

- Communicate attack method to team members

- Communicate interior fire conditions to team and IC

- Communicate rescue and ventilation needs with IC

- Advance to seat of fire and extinguish

- Team integrity is maintained (Sight, voice or physical contact)

- Maintain Situational awareness (Have egress established and protected) - Verbalize that routes of egress have been established and protected

5 Minutes

5- Demonstrate the correct procedures for control of a fire involving a flammable gas cylinder by cooling vapor space in tank and monitoring cylinder while approaching tank to close service line valve (9- firefighter team)

- Apply solid stream to vapor space of tank from maximum effective reach (If it can be accomplished safely, i.e. distance and location, until relief valve(s) close)

- Contents are identified

- Cylinder integrity is evaluated

- Confirm order with officer to approach tank to shut off valve

- Identify escape routes and safe havens

- Extend hoseline's to isolate control valve upwind and uphill

- Coordinated advance to cylinder

- Adjust nozzle patterns during advance for crew protection

- Try not to extinguish flame

- Close valve completely without breaking protective fog stream

- Coordinated retreat from cylinder

- Maintain nozzle patterns and cooling stream on vapor space during retreat

- Maintain team integrity and monitor situational awareness

- Continue monitoring pressure relief valves

20 Minutes

6- Protect evidence of fire cause and origin so that the evidence is properly protected from further disturbance.

- Identify the fire origin area

- Identify possible causes for fire ignition

- Identify types of evidence

- Preserve any evidence by leaving where found

- Establish barriers to protect the scene and evidence as found

- Notify command or fire investigator of findings

5 Minutes

RESCUE OPERATIONS

7- Demonstrate the following evolutions, which may be required to extricate an entrapped victim of a motor vehicle accident by displacing or removing:

A- Vehicle windshield (2 firefighter team)

- Conduct scene “Size up” and identify” Safety Zones” - Crib vehicle for safety

- Disconnect the vehicle battery

- Maintain victim safety during windshield removal

- Remove windshield completely

10 Minutes

B- Vehicle Roof (4 firefighter team)

- Conduct scene “Size up” and identify” Safety Zones” - Crib vehicle for safety

- Disconnect the vehicle battery

- Check for supplemental restraint safety system

- Remove side and rear windows as needed

- Maintain victim safety during roof removal

- Cut front vehicle roof posts and fold roof back or remove roof entirely

5 Minutes

C- Vehicle Door (2 firefighter team)

- Conduct scene “Size up” and identify” Safety Zones” - Crib vehicle for safety

- Disconnect the vehicle battery - Check for supplemental safety restraint systems

- Maintain victim safety during door removal

- Remove door

5 Minutes (Using powered equipment)

15 Minutes (Using hand tools)

D- Displace dashboard (2 firefighter team)

- Conduct scene “Size up” and identify” Safety Zones” - Crib vehicle for safety

- Disconnect the vehicle battery - Check for supplemental safety restraint systems

- Maintain victim safety during dashboard displacement

- Displace the dashboard

10 Minutes (Using powered equipment)

20 Minutes (Using hand tools)

PREVENTION, PREPAREDNESS, & MAINTENANCE

8- Prepare a fire safety survey in a private dwelling

- Contact Resident

- Explain purpose and benefit to resident

- Conduct survey, identify fire hazards

- Recommend appropriate resolutions to resident

- Discuss general fire safety information with them

- Conclude survey

- Complete inspection form

20 Minutes

9- Present fire safety information to station visitor or small groups

- Determine the audience and safety topic to be taught

- Conduct presentation according to lesson outline

- Return equipment and materials according to department policy

- Record information about the presentation in appropriate database

20 Minutes

10- Prepare a pre-incident survey

- Draw a plot plan

- Draw a floor plan

- Identify utility controls

- Identify means of egress

- Identify occupant information

- Identify occupancy or life safety hazards

- Provide recommendations to correct hazards

- Identify suppression systems and controls

- Use standard mapping symbols and abbreviations

30 Minutes

11- Demonstrate the service and maintenance of portable power plants and lighting equipment

- Check oil and gasoline for ‘full’ level

- Check air cleaner elements for debris

- Check spark plug and spark plug wire for deterioration and proper connection

- Check unit for loose bolts or screws

- Check electrical cords to ensure insulation is not damaged

- Check portable lights for damage

- Start unit and test operation of lighting equipment

- Document maintenance of equipment in logbook

5 Minutes

12- Demonstrate annual service test for fire hose (2-firefighter team) (150’ to 300’ of 1½” or larger hose)

- Connect hose sections together (check gaskets before connecting)

- Tighten connections between sections with spanner

- Connect test length to test valve (gate valve with ¼” hole drilled into center of gate)

- Attach nozzle to open end of hose line

- Fill hose with a pump pressure of 50 psi or to hydrant pressure

- Open nozzle and discharge all air from hose line

- Close nozzle

- Check hose for kinks and twists or leaking connections

- Mark hose jackets against each coupling

- Close test gate valve

- Increase pump pressure to the required test pressure (250 psi)

- Maintain test pressure for 5 minutes

- Slowly reduce pump pressure

- Close discharge valves and open nozzle to drain pressure from hose-line

- Check marks placed on the hose at the couplings

- Record the test results for each section of hose

10 Minutes

- Extrication Evolution2-member team 6:00 minutes

- Assess vehicle and associated hazards

- Establish safety zones

- Deploy fire extinguisher or handline

- Crib vehicle for safety

- Disconnect the vehicle battery

- Remove glass

- Check for supplemental safety restraint system

- Start/operate extrication equipment (Not to be used on vehicle for exam)

- Maintain victim safety

- Remove applicable doors

- Roll/jack dashboard

- Extinguishment of a Class B Fire 2-member team 8:00 minutes

- Give a brief size up

Unit on scene

Location

Type of building and construction

Current conditions

Declare a strategy

Initial actions

Identify side A

Establish Incident Command and Accountability and announce location

Name the incident

- Request additional resources if needed

- Transfer command

- Assemble foam application system if applicable

- Select correct concentration for required application if applicable

- Approach spill on air as part of coordinated team

- Demonstrate proper application techniques (Bank, Roll or Rain)

- Retreat from spill maintaining team integrity

- Identify the fire origin area

- Identify possible sources of fire ignitions verbalize

- Identify types of evidence verbalize

- Protect any evidence by leaving where found

- Establish barriers to protect the scene and evidence as found

- Notify command or investigator of findings

# HAZMAT

# HAZMAT AWARENESS:

HAZ MAT AWARENESS INITIATING REQUIRED NOTIFICATION – Page 42

- INITIATION REQUIRED NOTIFICATION

HAZMAT OPERATIONS:

# REPORTING AND DOCUMENTATION-Page 42-43

- DEMOSTRATE TRANSFERRING COMMAND

- INITIAL ACTION PLAN

- ICS 202 INCIDENT ACTION PLAN

DECONTAMINATION-Page 44-45

- DECONTAMINATION OF A VICTIM EMERGENCY

MULTIPLE DECONTAMINATION CORRIDOR SETUP

- DECONTAMINATION OF AN EMERGENCY RESPONDER

# PERSONAL PROTECTIVE EQUIPMENT-Page – 46-47

- SCBA IDENTIFCIATION

- SCBA DONNING

- SCBA DOFFING

- SPLASH SUIT DONNING

# PREFORMING DEFENSIVE CONTROL-Page- 48-49

- REMOTE VALVE SHUTTOFF

- FOAM APPLICATION

- DIKE

- DAM

- ABOSORBANT MATERIAL

- PRESERVATION OF EVIDENCE OF TERRORIST ATTACK

HAZMAT

HAZARDOUS MATERIALS AWARENESS

- Demonstrate initiating required notifications at a hazardous materials/WMD incident, given a hazardous materials/WMD incident. CONDITION: Given a scenario involving Haz-Mat/WMD incident including current edition ERG, SDS, shipping papers, approved communications equipment. • Initiate call to 911 or emergency dispatch. • Identify yourself. • Location of incident. • Explain the nature of the incident (Semi-tanker rollover, container leaking, amount of product being released). • Identify additional concerns; victims, injuries, involved in fire, etc. • Identify product involved/ being released. (ERG, SDS, Shipping papers) • Identify the initial isolation distance required. • Identify procedures for isolating and denying entry. 10 Minutes

HAZMAT

REPORTING AND DOCUMENTATION

1-Demonstrate procedures for establishing and transferring command at a Hazardous Materials/WMD incident, utilizing an incident management system

Given a brief size-up: - Unit on scene - Location - Nature of incident (i.e. Containers, location, nearby exposures, risks) - Current conditions (This is what I see) - Identify material. - Identify the isolation zones - Implement appropriate emergency operations - Implement site safety and control plan - Designate a safety officer - Establish Incident Command & Accountability and announce location - Name the Incident 7 Minutes

2-Brief assigned personnel so that the personnel are informed of specific tasks, standards, safety, operational, and special interest area considerations for the following:

Given one of two scenarios involving HazMat/WMD incidents, including a completed Incident Action Plan - Status of incident - Hazards identified - Tasks to be performed - Description of the site - Expected duration of the tasks - PPE requirements - Monitoring requirements - Describe emergency radio traffic procedures - Describe evacuation signal procedures 7 Minutes

HAZMAT

REPORTING AND DOCUMENTATION

3- Identify and complete the reporting and documentation requirements within the emergency response plan or IAP regarding PPE.

Given a scenario and a blank ICS 202 & ICS 204 or other documents used by the AHJ to meet FEMA and/or department SOG’s, document the type of PPE required by the scenario and special instructions. ICS 202 (General Safety Message). *Requires the use of PPE when exposures are expected. • Fill out the ICS 204 (Special Instructions). -Identify type of PPE required by scenario. -Identify when it is to be worn. -Reinforce compliance. • Verbalize if objectives are being met. -Conditions improving or not. -PPE appropriate. -Isolation zones adequate. • Verbalize adjustments as necessary. 10 Minutes (Time begins after scenario has been reviewed)

HAZMAT

DECONTAMINATION

4- Demonstrate Emergency Decontamination of a victim or responder by using water

- Select appropriate sit to minimize runoff contamination

- Responder directs victim on procedures (flush, strip, flush, cover)

- Rinse personnel starting from the head, working towards the feet

- Victim being decontaminated removes contaminated clothing as much as possible

- If absolutely necessary, decon team members touch outside of clothing being removed to avoid cross contamination

- Victim places contaminated articles in one designated area within contamination zone

- Victim is rinsed head to toe again

- Victim given protective clothing - Victim directed to exit warm zone for further evaluation

5 Minutes (2-firefighter team)

5- Demonstrate the setting up of a multiple station decontamination corridor

- Select appropriate site, up wind of incident, drainage towards “hot” zone

- Clearly marked entry point

- Tool drop

- Confinement of decon solutions and runoff water

- Wash station (may be multiple) - Rinse station (may be multiple) - SCBA drop/bottle exchange - PPE drop station(s) - Clearly marked exit point

10 Minutes (2-firefighter team)

6- Demonstrate decontamination of emergency responder NON-ENCAPSULATED SUIT: - Direct responder to enter at correct point of entry - Direct responder to drop tools in designated area - Direct responder to enter wash station - Wash responder starting from head - Decon personnel control run-off of wash station - Direct responder to drop SCBA in designated area without moving face-piece - Decon team members touch outside of clothing being removed to avoid contaminating responders - Remove contaminated PPE or clothing - Place contaminated articles in one designated area within corridor - Direct responder to remove face-piece - Direct responder to remove inner gloves - Direct responder to exit warm zone for further evaluation 15 Minutes (Two firefighter team) ENCAPSULATED SUIT: - Direct responder to enter entry point - Direct responder to drop tools in designated area - Direct responder to enter wash station - Wash responder from the head - Decon personnel control run-off of wash station - Remove contaminated PPE - Decon team member touch outside of clothing being removed to avoid contamination - Place contaminated articles in one designated area within corridor - Direct responder to drop SCBA in designated area without removing face-piece yet - Direct responder to remove face-piece - Direct responder to remove inner gloves - Direct responder to exit warm zone for further evaluation 10 Minutes (Two firefighter team)

PERSONAL PROTECTIVE EQUIPMENT

7- Identify and describe the function of each component of the Self-contained breathing apparatus (SCBA)

- Identify and describe the backpack/harness - Identify and describe the air cylinder – cylinder, valve, and pressure gauge - Identify and describe the regulator assembly – high pressure hose, low pressure alarm, main line valve, emergency by-pass valve, PASS device if equipped - Identify and describe the face-piece assembly – low pressure hose/mask-mounted regulator, exhalation valve, and head harness

5 Minutes

8- Demonstrate donning Self-Contained Breathing Apparatus (SCBA)

- Check SCBA cylinder pressure gauge for full level - Open cylinder valve fully, verbalize pressure - Check regulator and cylinder gauge – gauges should read within 100psi of each other - Don backpack and fasten/tighten all straps - Don face-piece, adjust head harness - Check seal, positive/negative pressure check - Attach supply hose to regulator or face-piece - Turn on PASS device if needed 1 Minute

9- Demonstrate doffing Self-Contained Breathing Apparatus (SCBA)

- Disconnect regulator from facepiece or low-pressure hose from regulator - Doff SCBA and place on ground, close cylinder valve - Relieve excess pressure from regulator, listen for low-pressure alarm - Turn off pass device, if equipped - Fully extend all straps on SCBA backpack - Fully extend all straps on facepiece - Inspect entire apparatus for cleanliness and damage

2 Minutes

10- Assist/Don emergency response personnel in donning chemical splash protective clothing and SCBA: (2-firefighter team)

Skills for person Donning Splash suit - Don hooded splash suit - Don boots - Don SCBA - Don facepiece - Check face seal - Don inner gloves - Don outer gloves - Don head protection Skills for person assisting in Donning of Splash suit - Assist donning splash suit - Assist donning boots - *Correctly tape suit to boots - *Completely tape hood of PPE to SCBA facepiece (do not cover field of vision on face piece) - Assist donning SCBA - *Tape front closure area and neck area - Assist donning inner gloves - Assist donning outer gloves - Assist responder to go on air - Fold al ends of tape (2” minimum) *Taping not required for encapsulating splash suits unless gloves and/or boots are not integrated into the suit.

6 Minutes (Encapsulating suit) 12 Minutes (Non-encapsulating suit)

11- Demonstrate stopping product release by closing remote valves

- Confirm location of remote valve - Approach valve location staying out of product - Open or close valve - Exit area maintaining team integrity - Evaluate whether shutting off remote valve stopped the leak (Must Verbalize)

2 Minutes (2-firefighter team)

12- Demonstrate the proper application of fire-fighting foam on a simulated hazardous materials spill

- Approach spill upwind/upgrade out of product - Demonstrate proper application techniques - Select one of three methods of foam application; Roll on, bank over and rain down - Do not direct stream directly into pool

4 Minutes (2-firefighter team)

13- Demonstrate construction of a dike to divert a spill

- Construct a dike 6 feet long and 6 inches high - Work ahead of spill, out of product - Two personnel build dike and one-member standby as safety - Divert spill - Evaluate whether the dike is constructed appropriately and accomplishes objective. (Must verbalize)

5 Minutes (3-firefighter team)

14- Demonstrate construction of a simple dam to control a spill

- Construct a dam 6- feet long and 8-12 inches high - Work ahead of spill, out of product - Two personnel build dam and one-member standby as safety - Control spill

5 Minutes (3-firefighter team)

15- Demonstrate the use of absorbent material to control a spill of a hazardous material

- Control or absorb spill by putting absorbent materials in place - Work ahead of spill, out of product - Two personnel put materials in place and one-member standby as safety

5 Minutes (3-firefighter team)

HAZMAT

PREFORMING DEFENSIVE CONTROL ACTIONS

PERFORMING DEFENSIVE CONTROL ACTIONS

16-Demonstrate proper evidence preservation for suspected criminal or terrorist acts. - Prevent individuals from altering or destroying physical evidence. - Restrict responder movement, location and activity around the evidence location. - Secure and mark evidence location (barrier tape/road cones). - Notify command immediately of findings including location and time. 5 Minutes

# STAFF

Captain Lee Monsen - Program Coordinator - EMT-Paramedic, Technical Rescue Team, West Valley City Fire Department

801-558-2492 wvfdmedic@msn.com

Captain Darin Monsen - EMT-Intermediate, HAZMAT Technician, Unified Fire Authority

801-560-9022 dmonsen@westvalleyfireacademy.com

Captain Brian Christensen - EMT-Intermediate, Technical Rescue Team, HAZMAT Technician, Unified Fire Authority

801-554-9226 brianc129@hotmail.com

Engineer Wade Rigby - EMT, Technical Rescue Team South Davis Metro Fire District

801-499-0447 waderigby@gmail.com

Website: wvfireacademy.com

West Valley Fire Academy LLC.